Project Brief: ESP32 Multi-Channel Light Show & Audio Controller

1. System Overview & Hardware Setup

- Microcontroller: ESP32 NodeMCU / Development Board (running FreeRTOS Task Watchdog Timer configured for stability).

- Audio Input: MAX9814 Microphone module connected to IO2 (ADC1) with ADC_11db attenuation.

- AC Output Channels (10-Channel AC Solid State / Relay Board):

  - IO Pins: {11, 12, 13, 14, 21, 38, 39, 40, 41, 42}

- Future Expansion / Unused 16-Channel Pinouts:

  - Note: Kept on record for your upcoming WS2811 addressable strip or secondary controller integration.

  - Recommended / reserved GPIO mapping for a 16-channel configuration typically utilizes safe digital output pins (e.g., standard GPIOs 4, 5, 16, 17, 18, 19, 22, 23, 25, 26, 27, 32, 33, 25, 27, etc., avoiding bootstrap pins like GPIO 0, 2, 5, 12, 15).

2. Web Server & User Interface (UI) Architecture

- Access Point (AP Mode):

  - SSID: ESP32-LightShow

  - Password: 12345678

  - Local Web Dashboard hosted directly on the ESP32 via HTTP web server (/, /ping, /settings, /update, /save).

- Preferences & Persistence:

  - Uses ESP32 Preferences library (EEPROM-like non-volatile storage) to save individual pattern settings, audio states, and ballistics parameters across reboots.

- UI Features:

  - Real-time connection monitoring status badge with auto-reconnect ping.

  - Responsive mobile-first dark mode layout.

  - Dynamic form control activation (enables/disables specific sliders depending on whether Audio mode or a specific Pattern is selected).

3. Implemented Display Patterns & Logic

- Moving Dot:

  - Bounces back and forth; reverses direction instantly upon detecting an audio beat.

  - Fully adjustable Qty of Dots (evenly distributed across channels) and Dot Size/Width (1 to 5 contiguous channels).

- Chase 1 to 10 & Chase 10 to 1:

  - Beat-driven directional chasers.

  - Supports independent Qty of Dots and Dot Size/Width with precise mathematical spacing across all 10 channels.

- VU Meters (4 Variations):

  - Standard VU Meter (1 to 10) with peak-hold decay indicator.

  - Reversed VU Meter (10 to 1) with peak-hold decay indicator.

  - Dual VU Meter (1 & 10 inward to Center) — Peak indicators removed per request.

  - Center-Out VU Meter (5 & 6 outward to 1 & 10) — Peak indicators removed per request.

4. Audio Processing & Ballistics Tuning

- Audio Filtering Modes:

  - Bass Beat Filter: Uses a software low-pass filter (RC filter math mapped via frequency cutoff slider from 0–100Hz).

  - Full Range: Direct unfiltered analog sample stream.

- Beat Detection & Prediction Engine:

  - Rolling window peak-to-peak amplitude analysis with dynamic thresholding (currentPeakToPeak > 120).

  - Built-in Beat Multiplier (1–9x) and Antifreeze Timer Delay (prevents lockups during quiet breaks).

- Configurable VU Ballistics:

  - Independent Attack Rate (e.g., 0.15 for fast, responsive rise) and Release Rate (e.g., 0.01 for slow, smooth, analog-lazy fall-off) adjustable live via sliders and saved to EEPROM.