Here is your updated and comprehensive Project Blueprint, incorporating your new screen option, core architecture insights, and the brilliant "Digital Twin" simulator concept.

You can copy and paste this complete version into your notes for tomorrow!

ESP32-S3 Wi-Fi Light Show Controller: Master Project Blueprint

1. System Architecture & Core Distribution

- Microcontroller: ESP32-S3 (Dual-Core hardware optimized).

  - Core 0: Continuous audio sampling, low-pass bass beat filtering, and background Wi-Fi/TCP/IP network stack processing.

  - Core 1: FastLED transmission, animations, pin sequencing, and lightweight web server handling (loop()).

- Outputs:

  - 10 Discrete Channels (Relays/Transistors on standard GPIOs).

  - 10 WS2811 Addressable LED Channels (Independent multi-node strips).

- Control Interface: Local web server hosted on the ESP32, accessed via iPhone over local Wi-Fi using mobile-optimized JavaScript buttons (fetch) for instant single-touch response.

2. 10-Channel Discrete Light Controller Specs

- Patterns Defined So Far:

  - Moving Dot (reverses on the bass beat)

  - Chase 1 to 10

  - Chase 10 to 1

  - VU Meter

  - Strobe ALL

  - Strobe Chase

  - Strobe Moving Dot

- Core Parameters:

  - Speed

  - Size

  - Music Response (Bass vs. Full Range response)

3. WS2811 LED Strip Controller Specs

- Status: Expanded capabilities compared to the 10-channel discrete lights.

- Parameters to Include: Color palettes, custom colors, brightness, tail length/fade effects, and unique pattern selections.

4. 5-Screen App UI / Architecture

- Screen 1: Audio Parameter Selection

  - Fine-tunes how the system listens to audio (gain, filter thresholds, bass vs. full-range selection).

- Screen 2: 10-Channel Display Selection & Parameters

  - Selects the active pattern and adjusts its unique settings (Speed, Size, Music Response).

- Screen 3 & 4: WS2811 Pattern Selection & Preset Library

  - Screen 3: Picks a WS2811 pattern and customizes its parameters. Saving adds it to the library.

  - Screen 4: The Show Library displaying all saved configurations. Each has a checkbox to include/exclude it from the active playlist pool.

- Screen 5: Master Playback & Show Control

  - Master Power: Switch the entire display ON or OFF.

  - Playlist Mode: Option to Randomize the playlist order or play sequentially.

  - Display Duration: Set the time duration (in seconds) that each checked configuration displays before automatically transitioning to the next.

5. Advanced Features & Future Roadmap

- Persistent Storage: Use ESP32 Preferences or LittleFS so saved presets and checkbox states survive power cycles.

- "Digital Twin" / Virtual Phone Simulator (Future Feature):

  - Emulate one of the WS2811 strips (100 LED pixels) directly on the iPhone browser.

  - Allows testing and previewing light patterns from the couch without turning on physical hardware.

  - Implementation Strategy: Use WebSockets for efficient, lightweight data streaming (sending raw RGB coordinate arrays instead of heavy HTTP polling) and offload pixel rendering to the iPhone's HTML5 Canvas to protect Core 1 performance.

PARAMETERS

The Following parameters are to be stored in EEPROM and called up each time on startup. The previous values should be retained but the option to change must be given. If a change is made, that change is retained for the next power up cycle.

- Parameter 1 : Audio Response

  - The display must react to the music ‘beat’ or range as required by the following parameter settings

    - On / Off Toggle –

      - When off, the display reacts to clock timing only

        - Clock Timing Delay Value default 1 second

      - When on, the display reacts as per the parameters

    - Text Box with default value 1000Hz

      - Any beats selected below the entered value in this box will advance the display

    - Multiplier with default value 2

      - The display will calculate the multiplied number of beats for each beat detected, and advance the display accordingly

    - Antifreeze time Delay default value 2 seconds

      - If a beat is not detected within the Antifreeze time Delay, the display will advance one step and repeat after each delay until the music is re-started.

- Parameter 2 : Channels (10 or 16)

  - Options for 10 or 16 Channel Controller. Pin outs need to be configured as per the following for each option

    - 10 Channel

      - 240V IO Channels - IO11,12, 13, 14, 21, 38, 39, 40, 41, 42

      - WS2811 IO Channels – IO10,9,8,18,17,16,15,7,6,5

      - Audio Input is on IO2

    - 16 Channel

      - WS2811 IO Channels – IO14,13,12,11,10,9,8,18,17,4,5,6,7,15,16,2

      - Audio Input is on IO1

- Parameter 3 : Colours (Halloween, Christmas or Disco)

  - Predominant Colour Options are preset as follows:

    - Halloween Colours restricted to:

      - Red, Neon Green, Deep Purple, Orange

    - Christmas Colours restricted to:

      - Red, Forest Green, Ice Blue, Gold

    - Disco Colours predominantly (but not restricted to):

      - Red, Green, Blue, Yellow, White

- Parameter 3 : Setup (Star, Grid, Line)

  - Refers to the hardware setup of the light show

    - Star – centre point with 10 or 16 arms where pixel 1 is at the centre

    - Grid – 10 or 16 columns where pixel 1 is at the bottom

    - Line – 10 or 16 lines where each strip has been placed end to end in a line

- Parameter 4: Wrap (Y/N)

  - When a display gets to the end of the led strip should it wrap around the beginning of the strip again or reverse direction

- Parameter 5: Text

  - For Grid setups only, Text may be added to share as a scrolling banner as part of the light show. This parameter contains the text to be displayed

Primary Display Options for 240V Channel Selection

- Moving Dot

- Chase 1 to 10

- Chase 10 to 1

- VU Meter

Primary Display Options for WS2811 Channels

- Moving Dot

- Knight Rider

- VU Meter

  - Colour Options

    - Green, Yellow, Red, with White Max Bar

    - Rainbow with White Max Bar

- Twin VU Meter

  - Colour Options

    - Green, Yellow, Red, with White Max Bar

    - Rainbow with White Max Bar

- Quad VU Meter

  - Colour Options

    - Green, Yellow, Red, with White Max Bar

    - Rainbow with White Max Bar

- Graphic Equaliser

  - Colour Options

    - Green, Yellow, Red, with White Max Bar

    - Rainbow with White Max Bar

WS2811 Parameters

- SPEED : Text Entry for Time value from 1 to 10 seconds

  - This determines the time taken for the chosen display to go from one end to the other (ie 100 pixels in 1 second)

- SPEED REFINED : Text Entry Range from 1 to 100%

  - This is applied to the SPEED parameter to further refine the time taken for the chosen display to go from one end to the other. (ie 100% = 100 pixels in 1 second)

- COL-1

  - This is the first colour of the chosen Parameter 3 Option

- COL-2

  - This is the second colour of the chosen Parameter 3 Option

- COL-3

  - This is the third colour of the chosen Parameter 3 Option

- COL-4

  - This is the fourth colour of the chosen Parameter 3 Option

- SIZE : (Text Box Entry)

  - Number of pixels used for the chosen display pattern

- EFFECTS : (Lookup)

  - When the chosen pattern responds to the ‘beat’, this effect will be displayed

- EFFECT SCALE : (Text Box Entry)

  - The number of pixels included to create the effect

- BRIGHTNESS : Text Box Entry (0 – 100%)

  - The brightness for the LED’s for this display

- MUSIC :

  - How the display advances (to the beat or full range of music as set in Parameter 1)

Effect Options

- Burst

  - As the display changes direction, a burst moving and decaying away from the direction of the dot should appear, such that it looks like part of the dot was shed in the violent change of direction. As the burst moves away from the dot it should decay in brightness until it disappears completely)

- Tail

  - The tail effect for Knight rider makes the display look like a long worm rather than a dot. The EFFECT SCALE will determine the length of the worm and SIZE will determine the length of pixels used for the knight rider moving dot

TABLE OF PARAMETERS for 10 CHANNEL 240V Lights

|  |  | SPEED | SIZE | MUSIC |
| --- | --- | --- | --- | --- |
| 1 | MOVING DOT | 0.01 - 1 | 1-10 | BASS |
| 2 | CHASE 1-10 | 0-10 | 1-10 | BASS |
| 3 | CHASE 10-1 | 0-10 | 1-10 | BASS |
| 4 | VU METER | X | X | BASS |

TABLE OF PARAMETERS for WS2811 addressable LED’s

RGBWB = RED,GREEN,BLUE,WHITE,RAINBOW