Audio Requirement:

Create the software required to accurately flash IO48 for 100ms to the beat of the disco music being played within a range of 80 to 180 BPM.

Silence must result in the LED remaining off.

Internally number the IO48 flashes 1 to 4 and repeat.

Every time IO48 is a 1 or 3 flash Channel 10 for 100ms

Create a variable called “BEAT” that can be used by other software that is only true when the beat hits. This should be timed for the middle of the beat rather than the leading edge.

Create a variable called “BEAT_STEP” that can be used by other software that is only true when the beats hit on counts 1 and 3. This should be timed for the middle of the beat rather than the leading edge.

Do not use internal timers to activate the LED’s, timers will drift out of time with the music and flash the LED’s at the wrong time.

Do use predictive timing (for one beat only) based on previous beat to beat timing.

When the music track is changed, IO48 must flash at the correct BPM of the new track being played as quickly as possible.

Unacceptable Factors:

Missed Beats – IO48 Must flash for every beat.

Timing Drift – Visible drift from the beat must be kept to an absolute minimum

Incorrect timing – failed project.